| Activity | Tractor Operation |
| --- | --- |
| Company | J&A Farming |
| Location | Irrigation Way, Widgelli, NSW |

DO NOT use this machine unless you have been trained in its safe operation and authorised.

| Protective Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

| Potential Hazards |  |
| --- | --- |
|  | - Traffic and moving vehicles and machinery - Pedestrians - Overhead power lines - Uneven and unstable ground - Flying debris - Hydraulic fluid under pressure - Hot surfaces - Combustible liquids |

Pre-Operation

- Only trained and authorised persons are to operate tractors. - All operators must read the Manufacturers Operators Manual. - If using attachments, read the Attachment Operators Manual and corresponding SWP. - Do not operate tractor while under the influence of drugs or alcohol. - Complete Pre-Start Check prior to each operation. - Report any faults or damage to Farm Manager prior to operation. - Identify overhead powerlines in areas of operation and ensure adequate clearance. - Identify and avoid uneven and unstable ground including ditches, holes and embankments.

Safe Operation

- Maintain three points of contact when climbing in or out of the tractor. - Always sit in the operators seat when starting the engine or operating controls. - Passengers are not to be carried. - Ensure there are no pedestrains around the tractor or the intended area of operation. - Before starting engine, ensure parking brake is engaged, the Power Take-Off (PTO) is disengaged and all levers are in neutral position. - Ensure you are wearing the seat belt before moving the tractor. - Keep all parts of the body within the cabin at all times. - Observe the farm speed limit of 40 km/hr. - Slow down when turning or traversing uneven ground. - If the tractor tips over, do not attempt to jump clear, remain seated, brace yourself and lean away from the point of impact.

Post-Operation

- Park on a firm and level surface wherever possible. If parking on sloping ground, park across the slope. - To park the tractor, bring it to a complete stop, disengage the PTO, lower all implements to the ground, place all controls in their neutral position, set the park brake and stop the engine. - Remove the key from the ignition and climb out of the tractor, maintaining three points of contact at all times. - Complete a quick post-operation inspection to detect any obvious signs of damage. Report any faults or damage to Farm Manager immediately. - Lock the cab door. Return the key to the farm office at the end of each day.

| Verification of Competence |  |  |  |
| --- | --- | --- | --- |
| I verify that I have observed the trainees named below operating the tractor in accordance with this Safe Work Procedure. Based on these observations I have made the following assessments of their competence: |  |  |  |
| Trainee | Trainer | Date | Competent |
|  |  |  | Yes No |
|  |  |  | Yes No |
|  |  |  | Yes No |
|  |  |  | Yes No |

| Protective Equipment Pictograms |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |